CE 482 - Design of Steel Structures

Term Project

Design of Industrial Building

Prepared by

Submission Date

21.01.2016

- Overview of Structure

Design of Industrial Building is to be constructed. The structure has 12 m spacing in both directions having total of 48 m by 36 m. Used structural members are columns, beams, purlins, and braces. Fixed ends and pinned ends are provided whenever it is thought to be suitable

Figure 1: Layout (grids) of the system

Figure 2: An overview from X-Z plane

Figure 3: An overview from Y-Z plane

2. Load Cases and Combinations

2.1. Load Cases

DL (Dead_load+Self_Weight): The summation of dead load and self-weight cases

Live_Load: Live load

Snow_Load: Undrifted snow case

Snow_Load_Left: Drifted snow on the left part of the roof

Snow_Load_Right: Drifted snow on the right part of the roof

Seismic_Load_X+: Seismic load in +x direction

Seismic_Load_X-: Seismic load in -x direction

Seismic_Load_Y+: Seismic load in +y direction

Seismic_Load_Y-: Seismic load in -y direction

Wind_Load_X+(- -): Wind in +x direction, loading on I part of the roof is suction

Wind_load_X+(- +): Wind in +x direction, loading on I part of the roof is compression

Wind_Load_X-(- -): Wind in -x direction, loading on I part of the roof is suction

Wind_Load_X-(- +): Wind in -x direction, loading on I part of the roof is compression

Wind_Load_Y +(-): Wind in +y direction, loading on roof is suction

Wind_Load_Y -(-): Wind in -y direction, loading on roof is suction

In Y direction we have assumed that the roof behaves as H part and there will be just suction throughout the roof.

2.2. Load Combinations

Load Combinations are shown in Appendix A.

2.3. Load Calculations

2.3.1. DL (Dead_load+Self_Weight):

Dead loads and 1.15*self weight of the members are summed. 1.15 factor is used to account for the connection weights. A capture of dead weights exerted on the system is provided below. Note that 0.6 kN/m of cover load is barely seen on the roof despite the fact that it exists.

Figure 4: Dead loads exerted on the system

2.3.2. Live Load

Figure 5: Live loads exerted on the system

2.3.3. Snow load Calculations:

In snow load calculations, 3 cases are considered: Undrifted case, Drifted (left) case, Drifted (right) case. Snow load calculations and its screenshots are provided below.

a) S1,2 = Ce * Ct * Sk * μ = 1 * 1 * 1.05 * 0.8 = 0.84 kPa

q=0.84*12=10.08 kN/m

b) S1 = Ce * Ct * Sk * μ = 1 * 1 * 1.05 * 0.8/2= 0.42 kPa , 5.04 kN/m

S2 = Ce * Ct * Sk * μ = 1 * 1 * 1.05 * 0.8 = 0.84 kPa  , 10.08 kN/m

c) S1 = Ce * Ct * Sk * μ = 1 * 1 * 1.05 * 0.8 = 0.84 kPa , 10.08 kN/m

S2 = Ce * Ct * Sk * μ = 1 * 1 * 1.05 * 0.8/2 = 0.42 kPa , 5.04 kN/m

Figure 6: Undrifted Snow loads exerted on the system

Figure 7: Drifted (Right) snow loads exerted on the system

Figure 8: Drifted (Left) snow loads exerted on the system

2.3.4. Wind load Calculations:

Vb = 28 m/s qb = 282 * 1.25 * 0.5 = 490 N/m2

Cr (z) ;

Kr = 0.19*(0.3/0.05)0.07= 0.215

z=20 m

Cr (z) = 0.215 * ln(20/0.3) = 0.58

Vm (z) = 28*0.9*1 = 25.33 m/s

Iv = 1/(ln(20/0.3)) = 0.238

qp(z) = [1+7*0.238]*1.25*0.5*25.332 = 1069.4 Pa =1.07 kPa

qp,e = 1.07  kPa and Cpe,10 table is shown below

- θ values are taken as 0, calculations are made for flat roof since pitch angle is lower than 5 degrees according to prEN 1991-1-4.

In X direction we have h/d=0.42, D and E values are found interpolating between given values from Table 7.1 prEN 1991-1-4 on page 37.

| X Direction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A | B | C | D | E | F | G | H | I |
| Cpe,10 | -1.2 | -0.8 | -0.5 | 0.723 | -0.345 | -1.8 | -1.2 | -0.7 | -0.2 |
| Cpe,10 | -1.2 | -0.8 | -0.5 | 0.723 | -0.345 | -1.8 | -1.2 | -0.7 | +0.2 |
| W (kN/m) | -15.4 | -10.3 | -6.4 | 9.3 | -4.43 | -23.1 | -15.4 | -9 | -2.6 |
| W (kN/m) | -15.4 | -10.3 | -6.4 | 9.3 | -4.43 | -23.1 | -15.4 | -9 | +2.6 |

In Y direction we have h/d=0.56, D and E values are found interpolating between given values from Table 7.1 prEN 1991-1-4 on page 37.

| Y Direction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A | B | C | D | E | F | G | H | I |
| Cpe,10 | -1.2 | -0.8 | -0.5 | 0.741 | -0.38 | -1.8 | -1.2 | -0.7 | -0.2 |
| Cpe,10 | -1.2 | -0.8 | -0.5 | 0.741 | -0.38 | -1.8 | -1.2 | -0.7 | +0.2 |
| W (kN/m) | -15.4 | -10.3 | -6.4 | 9.5 | -4.9 | -23.1 | -15.4 | -9 | -2.6 |
| W (kN/m) | -15.4 | -10.3 | -6.4 | 9.5 | -4.9 | -23.1 | -15.4 | -9 | +2.6 |

Figure 9: Wind in +x direction, loading on I part of the roof is suction

Figure 10: Wind in +x direction, loading on I part of the roof is compression

Figure 11: Wind in -x direction, loading on I part of the roof is suction

Figure 12: Wind in -x direction, loading on I part of the roof is compression

Figure 13: Wind in +y direction, loading on the roof is suction

Figure 14: Wind in -y direction, loading on the roof is suction

2.3.5. Seismic Load

Seismic loads are calculated in 4 different directions as +X, -X, +Y, -Y. Calculations are done according to prEN 1998-1:2003 (E). Evaluating the mass participating ratios table periods in the directions of x and y are taken as: Tx=0.62 sec, Ty=0.238 sec. And base shear forces are calculated as Fbx=409 kN & Fby=508 kN. These loads are distributed using spacing and placed as point loads on the nodes of the structure proportional to story heights. The applications of this procedure are provided below.

Figure 15: Modal Participating Mass Ratios Table

Figure 16: Seismic load in +X direction

Figure 17: Seismic load in -X direction

Figure 18: Seismic load in +Y direction

Figure 19: Seismic load in -Y direction

3. Computer Model

Loadings models and member sections are provided above and below respectively.

4. Design Calculations

4.1. Column Calculations

The column carrying the maximum axial load is analysed.

Global Buckling

- Strong axis

Figure 20: An overview of column HE400M

Ga=3131012+104100619720012

Gb=10 (recommended value)   K=2.2 is found from alignment chart

λ=kLr=2.2*6000178.8=73.82<4.71*2*105275=127

Fe=π2*Eλ2=π2*2*10573.822=362.23 MPa

Fcr=(0.658275362.23)*275=200.14 MPa

ϕPn=0.9*Fcr*Ag=0.9*200.14*32580103=5868 kN>464.2 kN applied force

- OKAY!

- K=1 in weak direction(braced)

λ=kLr=1*600077=77.92<4.71*2*105275=127

Fe=π2*Eλ2=π2*2*10577.922=325 MPa

Fcr=(0.658275325)*275=193 MPa

ϕPn=0.9*Fcr*Ag=0.9*193*32580103=5659 kN>464.2 kN applied force

- OKAY!

Shear Check

Cv=1 (rolled section)

ϕVn=0.9*0.6*Fy*Aw*Cv=0.9*0.6*275*21*298*1103=929.3 kN> 38 kN applied shear

Beam Column Check

PPc=464.25659=0.08 <0.2

P2Pc+MrxMcx+MryMcy≤1

This ratio is taken from SAP2000 as 0.436 which is less than 1

- OKAY!

4.2. Beam Calculations

Figure 21: An overview of beam HE700B

Yielding

∅*Mn=Mp=Fy*Zx=0.9*275*8327*103106=2060.93 kN.m>890.1 kN.m

Lateral Torsional Buckling

Cb=1.136 taken from SAP2000

Lb=12000 mm

Lp=1.76*ry*EF y=1.76*68.7*200000275=3260.75 mm

rts2=Iy*CwSx=104*14440*109*160607340*103=6560.85 mm2

rts=80.99 mm

Lr=1.95*rts*E0.7*Fy*J*cSx*h0+J*cSx*h02+6.76*0.7*FyE2

=1.95*80.99*2000000.7*275*830.9*104*17340*103*698+830.9*104*17340*103*6982+6.76*0.7*2752000002

Lr=8208.5 mm

Lp>Lr

Mn=Fcr*Sx

Fcr=Cb*π2*ELprts2*1+0.078*J*cSx*h0*Lprts2

Fcr=1.136 *π2*2000001200080.992*1+0.078*830.9*104*17340*103*698*1200080.992

Fcr=198.5 MPa

∅*Mn=0.9*198.5*7340*103106=1311.37 kN.m>890.1 kN.m

- LTB governs and the beam HEB700 is OKAY.

Shear Check

Cv=1 (rolled section)

ϕVn=0.9*0.6*Fy*Aw*Cv=0.9*0.6*275*17*582*1103=1469 kN> 38 kN applied shear

- OKAY!

4.3. Bracing Calculations

Buckling

TUBO-D-368x6.3 is used as bracing.

L=16.97 m

r=0.1279 m= 127.9 mm & Ag=7159 mm2 (taken from SAP2000)

k=1 (pin ends)

λ=kLr=1*16970127.9=132.68>4.71*2*105275=127

Fe=π2*Eλ2=π2*2*105132.682=112.13 MPa

Fcr=0.877*112.13=98.33 MPa

ϕPn=0.9*Fcr*Ag=0.9*98.33*7159103=633.6 kN>300.45kN applied force

- OKAY!

4.4. Purlin Calculations

Try UPN240

- 2 m spacing is selected from BorusanPro Catalog.

Cover load =0.2*36*48=345.6 kN

For 12 m purlin, distributed cover load of 0.4 kN/m is calculated.

For self-weight of the channel section 33.2*9.81/103 is distributed as 0.326 kN/m.

- DL=0.4+0.326=0.726 kN/m

Snow load of 1.68 kN/m is determined.

- SL=1.68 kN/m

COMB=1.2DL+1.6SL

=1.2*0.726+1.6*1.68=3.56 kN/m is determined.

Figure 22: Purlin loading

Mmax=w*L28=3.56*1228= 64 kN.m

Mmax=ϕMy=0.9*Fy*Sx  		     Sx≥MyFyϕ=64*106275*0.9=259*103mm2=259 cm2

Using UPN240 is OKAY since it has Sx= 300 cm2 > 259 cm2

- The resistance of UPN240 :

ϕM=0.9*Fy*Sx=0.9*275*300*103mm2106=74.25 kN.m> 64 kN.m   (OKAY!)

- Therefore, select UPN240 section for purlins.

5. Connection Details

5.1. Beam-to-Column Connection

Figure 23: Beam to column connection

Figure 24: Beam to column connection		Figure 25: Beam to column connection

Use M20 bolts, Grade 10.9, s275

V= 701 kN		Taken from SAP2000

M=701*200 =140.2*103 kN.mm

Vi=701/12=58.4 kN

k=0nYi2=2*802+1602+2402+3202+4002=704000 mm2

Ni=200*701*400704000=79.66 kN

Bolt capacity

V= 58.4 kN

Frv=58.4*103π*102=186 MPa

Fnt=0.75*900=675 MPa (tensile capacity without shear)

Fnv=0.45*1000=450MPa

F’nt=1.3*675-6000.75*450*186=546.8 MPa tensile capacity with shear

Rn= F’nt*Ab=546.8*3141000=171.7 kN    , ϕRn=0.75*171.7=128.8 kN

Ni=79.66<128.8    OKAY!

5.2. Bracing-to-Column Connection

Since bracing-to-column connection

Fnt'=1.3*Fnt-Fntϕ*Fnv*frv≤Fnt

Since the braces work under only tension or compression because they are pinned to the connection frv=0 resulting equation is Fnt'=1.3*Fnt

Fnt'=1.3*0.75*900=877 MPa

Bolt Area=(pi*20^2)/4=314 mm^2

R=877*314=275.4 kN/bolt

5 bolts are used 1376.9 kN>25 kN OKAY!

Figure 1= TUBO-D-368x6.3 brace connection, note that brace diameter is 368 mm while the plate diameter is 500 mm.

5.3Purlin-to-Beam Connection

UPN240 section is decided in previous calculations, and M20 bolt is used.

In this connection, it is assumed that the bolts work under shear force since the purlin works under flexure. Thus, only the shear resistance of M20 bolt is considered.

Fnt'=1.3*Fnt-Fntϕ*Fnv*frv≤Fnt

Since there is no tensile force on the bolts connecting the purlins and the shear legs, above formulation cannot be used. It is assumed to use 45 percent reduction in shear strength of bolts and the folllwing calculations are done.

Fnv=0.45.1000=450 MPa

Bolt Area=(pi*20^2)/4=314 mm^2

R=450*314/1000=141.3 kN/bolt

6. Conclusion

During load calculations, Eurocode (EN1990, EN1991-1-1, EN1991-1-3, EN1991-1-4, EN1998) is used, whereas AISC2010 (American) is used in steel design calculations. Bracing is added in the direction of Y axis of frames and moment releases are implemented in that direction. Bracing is added on the roof in order to have continuous load distributing throughout the structure. Moment frames are allowed in the other direction(X).

Total weight of the structure is 7958.184 kN, and 540.8 kg/m2 of steel is used.